# Lab 1: Connection of Network Devices Using Ethernet

## Preparation of Ethernet Cable with RJ45 Connectors

### Materials Required:

- Ethernet cable (Cat5e or Cat6)

- RJ45 connectors

- Crimping tool

- Wire stripper

- Cable tester

### Procedure:

#### Type 1: Straight-through Cable

- Cut the Ethernet cable to the desired length.s

- Strip about 1 inch of the outer jacket from both ends of the cable using the wire stripper.

- Untwist the pairs of wires and arrange them according to the T568B wiring standard:

- Pin 1: White/Orange

- Pin 2: Orange

- Pin 3: White/Green

- Pin 4: Blue

- Pin 5: White/Blue

- Pin 6: Green

- Pin 7: White/Brown

- Pin 8: Brown

- Trim the wires to ensure they are all the same length.

- Insert the wires into the RJ45 connector, ensuring each wire goes into its respective slot.

- Use the crimping tool to secure the RJ45 connector onto the cable.

- Repeat steps 2-6 for the other end of the cable.

- Test the cable using a cable tester to ensure proper connectivity.

#### Type 2: Crossover Cable

- Follow steps 1-4 from the straight-through cable procedure.

- For one end of the cable, arrange the wires according to the T568A wiring standard:

- Pin 1: White/Green

- Pin 2: Green

- Pin 3: White/Orange

- Pin 4: Blue

- Pin 5: White/Blue

- Pin 6: Orange

- Pin 7: White/Brown

- Pin 8: Brown

- For the other end of the cable, arrange the wires according to the T568B wiring standard:

- Pin 1: White/Orange

- Pin 2: Orange

- Pin 3: White/Green

- Pin 4: Blue

- Pin 5: White/Blue

- Pin 6: Green

- Pin 7: White/Brown

- Pin 8: Brown

- Trim the wires to ensure they are all the same length.

- Insert the wires into the RJ45 connectors, ensuring each wire goes into its respective slot.

- Use the crimping tool to secure the RJ45 connectors onto the cable.

- Test the cable using a cable tester to ensure proper connectivity.

### Connection Testing Using Cable Tester

- Connect one end of the cable to the transmitter unit of the cable tester.

- Connect the other end of the cable to the receiver unit of the cable tester.

- Turn on the cable tester and observe the results.

- For a straight-through cable, all pairs should show continuity in the same order.

- For a crossover cable, the transmit and receive pairs should be crossed.

  - Orders : 1-8 for straight-through and 1-3, 2-6, 3-1, 6-2 for crossover.

## Output : Straight

## Output : Crossover

## Results

- The cable tester indicates that all wires are properly connected and there are no faults.

- Visible Outputs are shown in images.